# HLC-213A Open Type Thermostat

1、Application

Widely used with the rotary evaporator cooling; with the reactor system thermostat, biological reactor temperature control; for the cold trap to provide a cooling source; distillation system cooling; for the laboratory to provide the central cooling water; analysis of instruments, such as viscometer external circulation, temperature control of the rheometer, etc.; ultrasonic crushing, disperser cooling, etc.; external open container temperature control and circulation.

2、Features

1. pressure suction pump: large flow, smooth water flow, no splash can withstand -80 ~ 300 ℃ temperature long time temperature work;

2. One-piece moulding: and liquid and vapour contact parts using high-performance stainless steel and high-performance materials;

3. centrifugal mixing technology: pressure suction pump has a jet jet mixing internal circulation technology, better temperature uniformity;

4. Imported temperature sensor: high-precision four-wire imported PT100 temperature sensor, single-point, two-point, three-point temperature calibration;

5. low liquid level protection: alarm can be light and sound at the same time to remind.

3、Technical parameters

| Model | HLC-213A |
| --- | --- |
| Item No. | 1032102001 |
| Tank volume (L) | 13 |
| Temperature control range (℃) | RT+10~100 |
| Temperature control precision (℃) | ±0.03 |
| Setting precision (℃) | ±0.1 |
| Display precision (℃) | ±0.01 |
| Heating power (W) | 2000 |
| Display temperature mode | Touch Panel |
| Setting mode | Touch screen |
| Safety protection | Low level protection, high temperature protection |
| Ambient temperature (℃) | 5~32 |
| Power supply voltage (V) | 200~230 |
| Circulation mode | Internal and External Circulation |
| Flow rate (L/min) | 22 |
| Lift(M)(Max/Rated) | 3.0/2.0 |
| Circulation interface | 10mm Pagoda Joint |
| Gross weight(kg) | 16 |
| Tank opening(mm)(WxDxH) | 155*135*150 |
| Overall Dimension(mm)(WxDxH) | 200*320*360 |

4、Packing List

| Name | Quantity |
| --- | --- |
| Main unit | 1 unit |
| DN8 connector | 2pcs |
| M16*1 nut | 2pcs |
| 8*12 heat preservation pipe | 1.8m*2pcs |
| Certificate of Conformity (with warranty card) | 1 copy |
| Instruction manual | 1 copy |